User’s Guide of Upgrade Tools

Preliminary

ONTENTS

1.	Upgrade Tools Description	2

1.1	Upgrade Tools	2

1.1.1	PC to BOX	2

1.1.2	BOX to BOX	2

1.1.3	USB ICE to BOX	2

2.	RS232 Upgrade	3

2.1	Requirement	3

2.2	Notice	3

2.3	Operation flow	3

3.	BOX to BOX Upgrade	3

3.1	Requirement	4

3.2	Notice	4

3.3	Operation flow	4

4.	USB ICE Upgrade	5

3.1	Requirement	4

3.2	Notice	4

3.3	Operation flow	4

# Upgrade Tools Description

This document can provide user how to use ALI upgrade tools. Including equipment requirement, limitation and operation flow.

## Upgrade Tools

ALI provides three kinds of tools to upgrade M3329 STB firmware.

### PC to BOX

User can upgrade firmware through standalone PC by RS232 cable.

### BOX to BOX

Upgrade firmware from another STB. This tool is established inside STB.

### USB ICE to BOX

Upgrade firmware from USB ICE through EJECT tag interface.

# RS232 Upgrade

User can upgrade firmware through standalone PC by RS232 cable.

## Requirement

### RS232 Cable: Connect PC and M3329 STB

### Upgrade_2M.exe: Load firmware and overwrite it to STB.

### M3329 compiler firmware(*.bin) file.

## Notice

### STB need full firmware inside flash. Because there is no flash driver in

Upgrade_2M.exe.

### User can upgrade different data individually. These data are shown below.

- ALL Firmware

- Main Code

- Video Background

- Audio Background

- Default satellite Database

### RS232 Cable is Female to Male

## Operation

Step 1: Connect PC and STB by RS232 cable.

Step 2: Execute Upgrade_2M.exe

Step 3: Browse needed firmware (*.bin) like below

Step 4: Choose upgrade type

- allcode: ALL Firmware

- maincode: Program Code

- videoback: Video Background

- radioback: Radio Background

- defaultdb: Default Database

# BOX to Box upgrade

## Requirement:

### RS232 Cable: Connect Master and Slave STB.

### Master STB: Source firmware inside is needed.

### Slave STB: Destination, platform which is needed upgrade.

## Notice:

### Slave STB: Boot Loader inside is enough. Don’t need full firmware inside.

### RS232 Cable must be Female to Female and cross by NULL modem.

## Operation flow:

Step 1: Open Tools Menu by Menu key, user will see a screen as illustrate below:

Step 2: We offer the functions for user to connecting receiver (Master) to the other receiver (Slave) via RS-232.

- Connect master receiver to slave receiver via RS-232. Enter “Software Upgrade” menu in master receiver and press “Volume Up/Volume Down” buttons to switch upgrade mode.

Upgrade modes include: allcode / maincode / radioback / defaultdb1 / userdb

- Moving highlight to “Start” and press “OK” button to start upgrade.

- Turn on slave receiver and it needn’t operate any setting.

- Press “Exit” button to save the parameters and draw back.

# USB ICE upgrade

## Requirement:

### USB ICE

### Driver of USB ICE

### Flashwr.exe : Navigate ICE to upgrade firmware.

### Sdram_config.txt: including definition of SDRAM size and SDRAM amount.

### m27fb_am29.abs: Flash write Driver.

### Binary file: product_fta_M3329.bin generated by compiler.

## Notice:

### USB ICE can upgrade firmware enen though there is no any firmware inside flash.

### Sdram_config.txt: Example

RAM_SIZE = 8M    // 8 M byte SDRAM

RAM_CHIP_NUM = 1 // One SDRAM Chip

## Operation flow

### Example:

Step 1: Copy product_fta_M3329.bin (generated after compiler) to the same folder with

flashwr.exe.

Step 2: Open DOS window

Step 3: Execute command

flashwr product_fta_M3329.bin -f sdram_config.txt